/*

- MushroomFarm Monitor - Production Version

*

- Features:

- - 4.0" TFT Display (480x320) on VSPI bus

- - SD Card Logging on HSPI bus (dual SPI for zero interference)

- - DS3231 RTC with web-based automatic time sync

- - Historical data buffer (600 entries for 7.5 min capacity)

- - WiFi AP mode for web interface and relay control

- - Receives sensor data from ESP32-C3 via HTTP POST

- - All 15 spectrometer readings generated by ESP32-C3

- - CSV logging with timestamped filenames

- - Treatment-based spectro data organization

*

- Pin Configuration:

- TFT (VSPI): MOSI=23, MISO=19, CLK=18, CS=5

- SD (HSPI):  MOSI=13, MISO=12, CLK=14, CS=15

- RTC (I2C):  SDA=21, SCL=22

- Sensors: Received from ESP32-C3 via WiFi (HTTP POST)

- Button: GPIO33

*/

#include <WiFi.h>

#include <WebServer.h>

#include <ArduinoJson.h>

#include <TFT_eSPI.h>

#include <SPI.h>

#include <Wire.h>

#include <string.h>

#include "RTClib.h"

#include "SD.h"

#include "Adafruit_MCP23X17.h"  // MCP23017 I2C GPIO expander

// TFT Display (4.0" 480x320) - Uses VSPI automatically

TFT_eSPI tft = TFT_eSPI();

// Separate SPI instance for SD card (HSPI)

SPIClass hspi(HSPI);

// RTC DS3231   RTC_DS3231 rtc;

bool rtcAvailable = false;

// MCP23017 GPIO Expander for relay control  Adafruit_MCP23X17 mcp;

bool mcpAvailable = false;

// Relay pin definitions on MCP23017

// GPA ports (Bank A) - Main devices

#define RELAY1 0   // GPA0 - Mist Maker 1

#define RELAY2 1   // GPA1 - Mist Maker 2

#define RELAY3 2   // GPA2 - Mist Maker 3

#define EXHAUST_FAN 3  // GPA3 - Exhaust Fan

#define SPRINKLER_PUMP 4  // GPA4 - Sprinkler Pump

#define LIGHTS 5   // GPA5 - Lights

// Humidity thresholds for mist makers   const float HUMIDITY_LOW = 75.0;  // Turn on mist maker below this   const float HUMIDITY_HIGH = 85.0; // Turn off mist maker above this

// Temperature threshold for exhaust fan   const float TEMP_EXHAUST_HIGH = 29.0; // Turn on exhaust fan at or above this temperature

const float TEMP_EXHAUST_LOW = 22.0;  // Turn on exhaust fan at

or below this temperature

// Relay states to prevent constant switching  bool mistMaker1State = false;  bool mistMaker2State = false;  bool mistMaker3State = false;  bool exhaustFanState = false; bool sprinklerPumpState = false;

bool lightsState = false;

// Manual override flags to prevent automatic control  bool mistMaker1ManualOverride = false;  bool mistMaker2ManualOverride = false;  bool mistMaker3ManualOverride = false; bool exhaustFanManualOverride = false; bool sprinklerPumpManualOverride = false;

bool lightsManualOverride = false;

// Sprinkler pump timing (10-second duration)   unsigned long sprinklerStartTime = 0;

const unsigned long SPRINKLER_DURATION = 30000; // 10 seconds

bool sprinklerActiveThisSession = false;

// RTC time adjustment for 2-second delay

const int RTC_DELAY_SECONDS = 2;

// Manila Time Zone (UTC+8) - for reference only

const long MANILA_OFFSET_HOURS = 8;

// SD Card logging on HSPI bus

#define SD_CS_PIN 15  // SD Card Chip Select pin (HSPI)

bool sdAvailable = false;   unsigned long lastLogTime = 0;

const unsigned long LOG_INTERVAL = 3600000; // 1 hour =

3,600,000 ms

// Time update for display   unsigned long lastTimeUpdate = 0;   const unsigned long TIME_UPDATE_INTERVAL = 1000; // Update time display every second for real-time clock

//  REMOVED: TFT pause system (not needed with dual SPI buses)

//  REMOVED: SD stabilization delays (dual SPI = instant reliability)

// Button for switching display modes   #define BUTTON_PIN 33   bool lastButtonState = HIGH;   bool currentButtonState = HIGH;   unsigned long lastDebounceTime = 0;

unsigned long debounceDelay = 50;

// Display modes

bool showSpectroMode = false;  // false = temp/humidity, true = spectro readings

bool displayNeedsUpdate = false;  // Flag to update display only when needed

bool displayInitialized = false;  // Flag to track if display structure is drawn

bool currentDisplayMode = false;   // Track current display mode to detect changes

bool hasReceivedData = false;  // Track if any sensor data has been received (for "No sensor data" message)

// Wi-Fi AP credentials   const char* ssid = "MushrooMan";

const char* password = "RABE2026qt";

// Web server on port 80

WebServer server(80);

// Max number of sensors

const int MAX_SENSORS = 10;

// Sensor timeout threshold (30 seconds)

// This ensures stale sensors are detected quickly and power-cycled if needed

const unsigned long SENSOR_TIMEOUT_MS = 30000; // 30 seconds

= 30000 ms

// Relay on MCP23017 to control ESP32-C3 AC power (GPA7 / A7)

#define RELAY_C3_POWER 7 // GPA7 (A7) - used to power-cycle

ESP32-C3 devices

// Relay cycle settings for C3 power: energize (open NC) for 5 seconds then restore   bool relayCycleActive = false;   unsigned long relayCycleStart = 0;   unsigned long lastRelayCycleMillis = 0;   const unsigned long RELAY_OFF_DURATION = 5000; // 5 seconds   const unsigned long RELAY_CYCLE_COOLDOWN_MS = 60000; //

60 seconds cooldown between cycles

// Expected C3 sensor IDs (for startup/monitoring) - if any of these are stale, trigger cycle

const char* expectedC3Ids[] = { "sensor1", "sensor2", "sensor3" };   const int EXPECTED_C3_COUNT = 3;

struct SensorData {

String id;     float temp;     float hum;     String lastUpdated;

unsigned long lastUpdateMillis;  // Track when data was last received

int spectro[6];     // up to 6 readings     int spectroCount;   // how many readings were received     bool dataChanged;   // flag to track if this sensor's data changed   };

SensorData sensors[MAX_SENSORS];   int sensorCount = 0;

//  OPTIMIZED: Historical data buffer stores all readings per 5-minute window

//  Reduced buffer size to fit in ESP32 RAM (typically 200KB available)

//  - Buffer size: 800 entries (enough for ~3-4 hours of data at 5-min intervals)

//  - Data is logged hourly, so this provides plenty of buffer capacity   //  - At 5-min intervals: ~216 entries/hour (36 DHT + 180 spectro)   const int MAX_HISTORY_ENTRIES = 800; // Reduced for memory

constraints

struct HistoricalReading {

char sensorId[12];        // Fixed-size char array (sensor1, sensor2, etc. max 8 chars)

char treatmentId[12];     // Fixed-size char array (treatment1-5 max

11 chars)     float temp;     float hum;

char dateTime[12];        // Fixed-size char array (DD/MM/YYYY = 10 chars)

char timeOnly[12];        // Fixed-size char array (HH:MM:SS AM = 11 chars)

int spectro[3];           // Store only the 3 readings for this treatment     int spectroCount;

int readingIndex;         // For spectro: which of the 3 readings in this

window (0, 1, or 2)     unsigned long timestamp;   // millis() when received     unsigned long windowId;    // 5-minute window identifier

bool isValid;

};

HistoricalReading dataHistory[MAX_HISTORY_ENTRIES];   int historyCount = 0;

// Helper function to calculate day of year manually   unsigned long calculateDayOfYear(int year, int month, int day) {

// Days in each month (non-leap year)

int daysInMonth[] = {31, 28, 31, 30, 31, 30, 31, 31, 30, 31, 30, 31};

// Check for leap year

bool isLeapYear = (year % 4 == 0 && year % 100 != 0) || (year %

400 == 0);     if (isLeapYear) {

daysInMonth[1] = 29; // February has 29 days in leap year

}

unsigned long dayOfYear = day;     for (int i = 0; i < month - 1; i++) {

dayOfYear += daysInMonth[i];

}

return dayOfYear;

}

// Helper function to get current 5-minute window ID

// Uses hour and minute to create a unique window identifier for each

5-minute period   unsigned long getCurrentWindowId() {

if (!rtcAvailable) {

// Fallback: use millis() divided by 5 minutes

return (millis() / 300000); // 5 minutes = 300000 ms

}

DateTime now = rtc.now();

// Create window ID: (hour * 12) + (minute / 5)

// This gives us 12 windows per hour (0-11), unique per hour     // Combined with date, this creates unique windows     unsigned long windowInHour = now.minute() / 5; // 0-11 (12 windows per hour)     unsigned long dayOfYear = calculateDayOfYear(now.year(), now.month(), now.day()); // Day of year (1-365/366)     // Combine: day of year * 12 windows/hour * 24 hours + current hour

* 12 + window in hour

// This creates a unique window ID that resets daily but is unique within the day

return (dayOfYear * 288) + (now.hour() * 12) + windowInHour; // 288

= 12 windows * 24 hours

}

// Function declarations

void displayTreatmentColumn(int columnIndex, int colWidth, int startY, int treatmentNum,

const char* sensorId, int startIdx, int endIdx, uint16_t

textColor = TFT_GREEN);   String generateLogFilename(bool isTempHumid);   bool createSDFile(String filename, String content);   void logTempHumidityData();   void logSpectroData();   bool testSDCardWrite();   void getSDCardInfo();   void listSDCardFiles();

void addToDataHistory(String sensorId, float temp, float hum, int* spectro, int spectroCount);   void clearDataHistory();   String getDateOnly();   String getTimeOnly();  void handleRoot();  void handleTimeSync();  void handleRelayControl();

void handleResetOverrides();

// Create sorted array of sensor indices for ordered display   void getSortedSensorIndices(int sortedIndices[], int &count) {     count = sensorCount;

// Initially fill with sensor indices     for (int i = 0; i < sensorCount; i++) {

sortedIndices[i] = i;

}

// Simple bubble sort based on sensor ID     for (int i = 0; i < sensorCount - 1; i++) {       for (int j = 0; j < sensorCount - i - 1; j++) {         // Compare sensor IDs (sensor1, sensor2, etc.)         if (sensors[sortedIndices[j]].id.compareTo(sensors[sortedIndices[j

+ 1]].id) > 0) {           // Swap indices           int temp = sortedIndices[j];           sortedIndices[j] = sortedIndices[j + 1];

sortedIndices[j + 1] = temp;

}

}

}

}

// Find index of sensor by ID   int findSensorIndex(const String& id) {     for (int i = 0; i < sensorCount; i++) {

if (sensors[i].id == id) return i;

}     return -1;

}

// Check if sensor data is stale (hasn't been updated recently)  bool isSensorStale(int sensorIndex) {

if (sensorIndex < 0 || sensorIndex >= sensorCount) return true;    // Check if sensor has never been updated    if (sensors[sensorIndex].lastUpdateMillis == 0) return true;    unsigned long timeSinceUpdate = millis() - sensors[sensorIndex].lastUpdateMillis;

return timeSinceUpdate > SENSOR_TIMEOUT_MS;

}

// Remove stale sensors from active display (clear their data)   void clearStaleSensors() {     for (int i = 0; i < sensorCount; i++) {       if (isSensorStale(i)) {         // Clear stale sensor data         sensors[i].temp = 0.0;         sensors[i].hum = 0.0;         sensors[i].spectroCount = 0;         sensors[i].dataChanged = true; // Mark for display update

Serial.println(" Cleared stale sensor: " + sensors[i].id);

}

}

}

// Get formatted date and time string (12-hour format with AM/PM)

String getDateTime() {     if (rtcAvailable) {

DateTime now = rtc.now();

// Use RTC time directly (assuming RTC is already set to Manila time)       int hour12 = now.hour();

String ampm = "AM";

// Convert 24-hour to 12-hour format       if (hour12 == 0) {         hour12 = 12; // Midnight = 12 AM       } else if (hour12 > 12) {         hour12 -= 12; // PM hours         ampm = "PM";       } else if (hour12 == 12) {

ampm = "PM"; // Noon = 12 PM

}

char dateTime[25];

sprintf(dateTime, "%02d/%02d/%04d %02d:%02d:%02d %s",                now.day(), now.month(), now.year(),                hour12, now.minute(), now.second(), ampm.c_str());       return String(dateTime);     } else {       return "RTC Not Available";

}

}

// Get short time string for sensor updates (12-hour format)

String getTimeStamp() {     if (rtcAvailable) {

DateTime now = rtc.now();       // Use RTC time directly (already in Manila time)

int hour12 = now.hour();

String ampm = "AM";

// Convert 24-hour to 12-hour format       if (hour12 == 0) {         hour12 = 12; // Midnight = 12 AM       } else if (hour12 > 12) {         hour12 -= 12; // PM hours         ampm = "PM";       } else if (hour12 == 12) {

ampm = "PM"; // Noon = 12 PM

}

char timeStamp[12];

sprintf(timeStamp, "%02d:%02d:%02d%s", hour12, now.minute(), now.second(), ampm.c_str());

return String(timeStamp);

} else {

// Use system uptime in readable format       unsigned long totalSeconds = millis() / 1000;       unsigned long hours = totalSeconds / 3600;       unsigned long minutes = (totalSeconds % 3600) / 60;       unsigned long seconds = totalSeconds % 60;       char timeStamp[10];

sprintf(timeStamp, "%02lu:%02lu:%02lu", hours, minutes, seconds);       return String(timeStamp);

}

}

// Get temperature from DS3231 (bonus feature)

float getRTCTemperature() {     if (rtcAvailable) {       return rtc.getTemperature();

} else {       return 0.0;

}

}

//  NEW: Get date only (for CSV format)   String getDateOnly() {     if (rtcAvailable) {

DateTime now = rtc.now();       char dateOnly[12];

sprintf(dateOnly, "%02d/%02d/%04d", now.day(), now.month(), now.year());       return String(dateOnly);     } else {       return "No_Date";

}

}

//  NEW: Get time only (for CSV format)     String getTimeOnly() {     if (rtcAvailable) {

DateTime now = rtc.now();       // Convert to 12-hour format       int hour12 = now.hour();

String ampm = "AM";

if (hour12 == 0) {         hour12 = 12; // Midnight = 12 AM       } else if (hour12 > 12) {         hour12 -= 12; // PM hours         ampm = "PM";       } else if (hour12 == 12) {

ampm = "PM"; // Noon = 12 PM

}

char timeOnly[12];

sprintf(timeOnly, "%02d:%02d:%02d %s", hour12, now.minute(), now.second(), ampm.c_str());       return String(timeOnly);     } else {

unsigned long totalSeconds = millis() / 1000;       unsigned long hours = totalSeconds / 3600;       unsigned long minutes = (totalSeconds % 3600) / 60;       unsigned long seconds = totalSeconds % 60;

char timeOnly[12];

sprintf(timeOnly, "%02lu:%02lu:%02lu", hours, minutes, seconds);

return String(timeOnly);

}

}

//  OPTIMIZED: Add reading to historical buffer (only latest per 5minute window)

void addToDataHistory(String sensorId, float temp, float hum, int* spectro, int spectroCount) {

unsigned long currentWindow = getCurrentWindowId();

// Convert sensorId String to char array for comparison     char sensorIdBuf[16];

sensorId.toCharArray(sensorIdBuf, sizeof(sensorIdBuf));

// For DHT22 sensors: store latest reading per sensor per window     if (spectroCount == 0 || sensorId == "sensor1" || sensorId ==

"sensor2" || sensorId == "sensor3") {

// Check if we already have an entry for this sensor in this window

int existingIndex = -1;       for (int i = 0; i < historyCount; i++) {         if (dataHistory[i].isValid &&              strcmp(dataHistory[i].sensorId, sensorIdBuf) == 0 &&              dataHistory[i].windowId == currentWindow &&             strlen(dataHistory[i].treatmentId) == 0) {           existingIndex = i;

break;

}

}

if (existingIndex >= 0) {         // Update existing entry with latest reading         HistoricalReading* entry = &dataHistory[existingIndex];         entry->temp = temp;         entry->hum = hum;         String dateStr = getDateOnly();         String timeStr = getTimeOnly();

strncpy(entry->dateTime, dateStr.c_str(), sizeof(entry->dateTime) -

1);         entry->dateTime[sizeof(entry->dateTime) - 1] = '\0';         strncpy(entry->timeOnly, timeStr.c_str(), sizeof(entry->timeOnly) -

1);

entry->timeOnly[sizeof(entry->timeOnly) - 1] = '\0';         entry->timestamp = millis();         // Don't update spectro for DHT-only entries

} else {

// Create new entry

if (historyCount >= MAX_HISTORY_ENTRIES) {

Serial.println(" History buffer full - removing oldest entries");

// Remove oldest 20% of entries

int removeCount = MAX_HISTORY_ENTRIES / 5;           for (int i = 0; i < historyCount - removeCount; i++) {

dataHistory[i] = dataHistory[i + removeCount];

}

historyCount -= removeCount;

}

HistoricalReading* entry = &dataHistory[historyCount];         strncpy(entry->sensorId, sensorIdBuf, sizeof(entry->sensorId) - 1);         entry->sensorId[sizeof(entry->sensorId) - 1] = '\0';         entry->treatmentId[0] = '\0'; // Empty for DHT22 sensors         entry->temp = temp;         entry->hum = hum;         String dateStr = getDateOnly();         String timeStr = getTimeOnly();

strncpy(entry->dateTime, dateStr.c_str(), sizeof(entry->dateTime) -

1);         entry->dateTime[sizeof(entry->dateTime) - 1] = '\0';         strncpy(entry->timeOnly, timeStr.c_str(), sizeof(entry->timeOnly) -

1);

entry->timeOnly[sizeof(entry->timeOnly) - 1] = '\0';         entry->spectroCount = 0;         entry->timestamp = millis();         entry->windowId = currentWindow;         entry->isValid = true;

historyCount++;

}

}

// For Spectrometer data: store ALL 3 readings per treatment per

window

if (spectroCount > 0) {

// Process treatments based on sensor ID and spectro values

struct TreatmentData {         String treatmentId;         int startIdx;

};

TreatmentData treatments[5];

int treatmentCount = 0;

if (sensorId == "sensor1" && spectroCount >= 3) {         treatments[treatmentCount].treatmentId = "treatment1";         treatments[treatmentCount].startIdx = 0;         treatmentCount++;         if (spectroCount >= 6) {           treatments[treatmentCount].treatmentId = "treatment2";           treatments[treatmentCount].startIdx = 3;           treatmentCount++;

}

} else if (sensorId == "sensor2" && spectroCount >= 3) {         treatments[treatmentCount].treatmentId = "treatment3";         treatments[treatmentCount].startIdx = 0;         treatmentCount++;         if (spectroCount >= 6) {

treatments[treatmentCount].treatmentId = "treatment4";           treatments[treatmentCount].startIdx = 3;           treatmentCount++;

}

} else if (sensorId == "sensor3" && spectroCount >= 3) {         treatments[treatmentCount].treatmentId = "treatment5";         treatments[treatmentCount].startIdx = 0;         treatmentCount++;

}

// Store each treatment's reading (we receive multiple POSTs per window, each with 3 values)       for (int t = 0; t < treatmentCount; t++) {         String treatmentId = treatments[t].treatmentId;         int startIdx = treatments[t].startIdx;         char treatmentIdBuf[16];

treatmentId.toCharArray(treatmentIdBuf, sizeof(treatmentIdBuf));

// Count how many readings we already have for this treatment in this window         int existingCount = 0;         for (int i = 0; i < historyCount; i++) {           if (dataHistory[i].isValid &&                strcmp(dataHistory[i].treatmentId, treatmentIdBuf) == 0 &&                dataHistory[i].windowId == currentWindow) {

existingCount++;

}

}

// Determine which reading index this is (0, 1, or 2)

int readingIndex = existingCount;

// Store all 3 readings that arrive in this POST (we can receive up

to 3 POSTs per treatment per window)         if (readingIndex < 3) {           // Create new entry for this reading set

if (historyCount >= MAX_HISTORY_ENTRIES) {

Serial.println(" History buffer full - removing oldest entries");             int removeCount = MAX_HISTORY_ENTRIES / 10; // Remove

10% when full

for (int i = 0; i < historyCount - removeCount; i++) {

dataHistory[i] = dataHistory[i + removeCount];

}

historyCount -= removeCount;

}

HistoricalReading* entry = &dataHistory[historyCount];           strncpy(entry->sensorId, sensorIdBuf, sizeof(entry->sensorId) -

1);           entry->sensorId[sizeof(entry->sensorId) - 1] = '\0';           strncpy(entry->treatmentId, treatmentIdBuf, sizeof(entry-

>treatmentId) - 1);           entry->treatmentId[sizeof(entry->treatmentId) - 1] = '\0';           entry->temp = 0.0; // Not used for spectro-only entries

entry->hum = 0.0;           String dateStr = getDateOnly();           String timeStr = getTimeOnly();

strncpy(entry->dateTime, dateStr.c_str(), sizeof(entry->dateTime)

- 1);

entry->dateTime[sizeof(entry->dateTime) - 1] = '\0';           strncpy(entry->timeOnly, timeStr.c_str(), sizeof(entry->timeOnly) -

1);

entry->timeOnly[sizeof(entry->timeOnly) - 1] = '\0';           entry->spectroCount = 3;           entry->readingIndex = readingIndex;           entry->timestamp = millis();           entry->windowId = currentWindow;           entry->isValid = true;

// Store the 3 readings for this treatment from this POST           for (int i = 0; i < 3 && (startIdx + i) < spectroCount; i++) {

entry->spectro[i] = spectro[startIdx + i];

}

historyCount++;

Serial.println(" Stored " + treatmentId + " reading set " +

String(readingIndex + 1) + "/3 for window " + String(currentWindow));

} else {

Serial.println(" Window " + String(currentWindow) + " already has

3 readings for " + treatmentId + " - skipping");

}

}

}

}

//  NEW: Clear historical data buffer after logging

void clearDataHistory() {

Serial.println(" Clearing historical data buffer (" + String(historyCount)

+ " entries)");     historyCount = 0;     // Mark all entries as invalid     for (int i = 0; i < MAX_HISTORY_ENTRIES; i++) {       dataHistory[i].isValid = false;

}

}

// Manual RTC time setting function

void setRTCTime(int year, int month, int day, int hour, int minute, int

second) {

if (rtcAvailable) {

DateTime newTime = DateTime(year, month, day, hour, minute, second);       rtc.adjust(newTime);

Serial.println(" RTC time updated to: " + getDateTime());

} else {

Serial.println(" RTC not available - cannot set time");

}

}

// Test SD card write capability (simplified)   bool testSDCardWrite() {

File testFile = SD.open("/test.txt", FILE_WRITE);

if (testFile) {       testFile.println("Test OK");       testFile.close();       SD.remove("/test.txt");       Serial.println(" SD card write test passed");

return true;

}

Serial.println(" SD card write test failed");     return false;

}

// Get SD card basic information   void getSDCardInfo() {     uint8_t cardType = SD.cardType();     uint64_t totalBytes = SD.totalBytes() / (1024 * 1024);

uint64_t usedBytes = SD.usedBytes() / (1024 * 1024);

Serial.println(" SD Card: " + String(totalBytes) + "MB, " +

String(usedBytes) + "MB used");

if ((totalBytes - usedBytes) < 10) {       Serial.println(" Low space - consider cleanup");

}

}

// Initialize SD Card with HSPI bus   bool initializeSDCard() {

Serial.println(" Initializing SD Card on HSPI bus...");

// Initialize HSPI for SD card     hspi.begin(14, 12, 13, 15); // SCK, MISO, MOSI, CS

if (!SD.begin(SD_CS_PIN, hspi)) {

Serial.println(" SD Card initialization failed!");       return false;

}

if (SD.cardType() == CARD_NONE) {       Serial.println(" No SD card detected");

return false;

}

// Basic diagnostics

getSDCardInfo();

// Test write capability     if (!testSDCardWrite()) {       Serial.println(" SD Card write test failed!");

return false;

}

Serial.println(" SD Card operational on HSPI bus");     listSDCardFiles();

return true;

}

// List files on SD card   void listSDCardFiles() {     File root = SD.open("/");

if (!root) return;

int fileCount = 0;     File file = root.openNextFile();

while (file) {       if (!file.isDirectory()) {         fileCount++;

Serial.println(" " + String(file.name()) + " (" + String(file.size()) + " bytes)");

}       file.close();       file = root.openNextFile();

}

root.close();

Serial.println(" Total: " + String(fileCount) + " files");

}

//  NEW: Generate date-time based filename for CSV logs

String generateLogFilename(bool isTempHumid) {

String filename;

if (rtcAvailable) {

DateTime now = rtc.now();

// Create filename format: DDMmmYYYY-HHMM-

(temp/spectro).csv

// Example: 25Sep2025-1430-temp.csv

String months[] = {"Jan", "Feb", "Mar", "Apr", "May", "Jun",

"Jul", "Aug", "Sep", "Oct", "Nov", "Dec"};

char dateTime[20];       sprintf(dateTime, "%02d%s%04d-%02d%02d",

now.day(),                months[now.month()-1].c_str(),

now.year(),                now.hour(),

now.minute());

if (isTempHumid) {

filename = "/" + String(dateTime) + "-temp.csv";

} else {

filename = "/" + String(dateTime) + "-spectro.csv";

}

Serial.println(" Generated timestamped filename: " + filename);

} else {

// Fallback to uptime-based naming when no RTC

unsigned long uptimeMinutes = millis() / 60000;

if (isTempHumid) {

filename = "/uptime" + String(uptimeMinutes) + "-temp.csv";

} else {

filename = "/uptime" + String(uptimeMinutes) + "-spectro.csv";

}

Serial.println(" Generated uptime-based filename: " + filename);

}

return filename;

}

// Ultra-simple SD file creation (EXACT copy of working sd_logging_test.ino approach)   bool createSDFile(String filename, String content) {     Serial.println(" Creating file (sd_logging_test.ino approach): " + filename);

// EXACT same approach as working isolated test - no complex operations needed

// since TFT operations are now paused during SD logging!

File file = SD.open(filename, FILE_WRITE);

if (!file) {

Serial.println(" Cannot create file: " + filename);

return false;

}

// Write content exactly like working test

file.print(content);

file.close();

Serial.println(" File written (sd_logging_test.ino approach)");

return true;

}

// Get mistmaker status for a specific sensor

String getMistmakerStatus(String sensorId) {    // Map sensors to mistmakers based on control logic    if (sensorId == "sensor1") {      return mistMaker1State ? "ON" : "OFF";    } else if (sensorId == "sensor2") {      return mistMaker2State ? "ON" : "OFF";    } else if (sensorId == "sensor3") {

return mistMaker3State ? "ON" : "OFF";

} else {

return "N/A";  // No mistmaker associated with this sensor

}

}

//  OPTIMIZED: Log historical temperature and humidity data (only

DHT22 entries)   void logTempHumidityData() {     String filename = generateLogFilename(true);

Serial.println(" Creating temp/humidity log: " + filename + " (" +

String(historyCount) + " total entries)");

// Build CSV content with new format: Date | Time | SensorName |

Temp | Humidity | Status

String content = "Date,Time,Sensor,Temperature,Humidity,Status\n";

int logCount = 0;

for (int i = 0; i < historyCount; i++) {

if (dataHistory[i].isValid && strlen(dataHistory[i].treatmentId) == 0) {         // Only log DHT22 entries (treatmentId is empty)         content += String(dataHistory[i].dateTime) + ",";         content += String(dataHistory[i].timeOnly) + ",";         content += String(dataHistory[i].sensorId) + ",";         content += String(dataHistory[i].temp, 2) + ",";         content += String(dataHistory[i].hum, 2) + ",";         content += getMistmakerStatus(String(dataHistory[i].sensorId)) +

"\n";         logCount++;

}

}

Serial.println(" Content prepared: " + String(content.length()) + " bytes for " + String(logCount) + " readings");

// Single file operation     if (createSDFile(filename, content)) {

Serial.println(" Temperature/Humidity data logged successfully!");

Serial.println("   File: " + filename);

Serial.println("   Historical readings: " + String(logCount));

} else {

Serial.println(" Temperature/Humidity logging failed");

}

}

//  OPTIMIZED: Log historical spectro data grouped by window with treatment columns   void logSpectroData() {

String filename = generateLogFilename(false);

Serial.println(" Creating spectro log: " + filename + " (" +

String(historyCount) + " total entries)");

// Build CSV content with new format:

Date,Time,Treatment1,Treatment2,Treatment3,Treatment4,Treatment5     // Each treatment column contains 3 readings as space-separated

values: "val1 val2 val3"

String content =

"Date,Time,Treatment1,Treatment2,Treatment3,Treatment4,Treatment

5\n";

// Group readings by window ID

// For each window, collect all 3 readings for each treatment     struct WindowData {       unsigned long windowId;       String dateTime;

String timeOnly;

String treatments[5][3];  // 5 treatments, each with up to 3 readings       int treatmentCounts[5];   // How many readings we have for each treatment (0-3)       bool hasData;

};

// Find all unique windows

unsigned long uniqueWindows[200];  // Max 200 unique windows

(reduced for memory)

int uniqueWindowCount = 0;

for (int i = 0; i < historyCount; i++) {

if (dataHistory[i].isValid && strlen(dataHistory[i].treatmentId) > 0) {         unsigned long winId = dataHistory[i].windowId;         bool found = false;         for (int j = 0; j < uniqueWindowCount; j++) {           if (uniqueWindows[j] == winId) {             found = true;             break;

}

}

if (!found && uniqueWindowCount < 200) {

uniqueWindows[uniqueWindowCount++] = winId;

}

}

}

int logCount = 0;

// Process each unique window     for (int w = 0; w < uniqueWindowCount; w++) {

unsigned long currentWinId = uniqueWindows[w];

// Initialize window data structure       WindowData winData;       winData.windowId = currentWinId;

winData.hasData = false;       for (int t = 0; t < 5; t++) {         winData.treatmentCounts[t] = 0;         for (int r = 0; r < 3; r++) {

winData.treatments[t][r] = "";

}

}

// Collect all readings for this window       for (int i = 0; i < historyCount; i++) {

if (dataHistory[i].isValid &&              strlen(dataHistory[i].treatmentId) > 0 &&              dataHistory[i].windowId == currentWinId &&

dataHistory[i].spectroCount >= 3) {

// Extract treatment number (1-5)           String treatmentId = String(dataHistory[i].treatmentId);           int treatmentNum = 0;           if (treatmentId == "treatment1") treatmentNum = 0;           else if (treatmentId == "treatment2") treatmentNum = 1;           else if (treatmentId == "treatment3") treatmentNum = 2;           else if (treatmentId == "treatment4") treatmentNum = 3;           else if (treatmentId == "treatment5") treatmentNum = 4;

if (treatmentNum >= 0 && treatmentNum < 5) {             int readingIdx = dataHistory[i].readingIndex;             if (readingIdx >= 0 && readingIdx < 3) {               // Store this reading set's 3 values at the appropriate index

// Each reading set contains 3 values, and we can have up to

3 reading sets per treatment

// For CSV output, we'll combine all 3 reading sets into one

row

// Store the 3 values from this reading set               if (winData.treatmentCounts[treatmentNum] == 0) {

// First reading set - initialize                 winData.treatments[treatmentNum][0] = String(dataHistory[i].spectro[0]);                 winData.treatments[treatmentNum][1] = String(dataHistory[i].spectro[1]);                 winData.treatments[treatmentNum][2] = String(dataHistory[i].spectro[2]);

winData.treatmentCounts[treatmentNum] = 1;               } else if (winData.treatmentCounts[treatmentNum] == 1) {

// Second reading set - append                 winData.treatments[treatmentNum][0] += " " + String(dataHistory[i].spectro[0]);

winData.treatments[treatmentNum][1] += " " + String(dataHistory[i].spectro[1]);

winData.treatments[treatmentNum][2] += " " + String(dataHistory[i].spectro[2]);

winData.treatmentCounts[treatmentNum] = 2;               } else if (winData.treatmentCounts[treatmentNum] == 2) {

// Third reading set - append                 winData.treatments[treatmentNum][0] += " " + String(dataHistory[i].spectro[0]);

winData.treatments[treatmentNum][1] += " " + String(dataHistory[i].spectro[1]);

winData.treatments[treatmentNum][2] += " " + String(dataHistory[i].spectro[2]);

winData.treatmentCounts[treatmentNum] = 3;

}

// Store date/time from first reading we encounter               if (!winData.hasData) {                 winData.dateTime = String(dataHistory[i].dateTime);                 winData.timeOnly = String(dataHistory[i].timeOnly);                 winData.hasData = true;

}

}

}

}

}

// Write row for this window       if (winData.hasData) {         content += winData.dateTime + ",";

content += winData.timeOnly + ",";

// Write each treatment's readings

// Each treatment column contains all 3 reading sets (9 values total: 3 sets × 3 values)

// Format: "val1_set1 val2_set1 val3_set1 val1_set2 val2_set2 val3_set2 val1_set3 val2_set3 val3_set3"

for (int t = 0; t < 5; t++) {           if (winData.treatmentCounts[t] > 0) {             // Combine all reading sets for this treatment             String treatmentData = winData.treatments[t][0] + " " +                                     winData.treatments[t][1] + " " +                                     winData.treatments[t][2];

content += "\"" + treatmentData + "\"";

} else {

// No readings for this treatment in this window

content += "\"\"";

}

if (t < 4) content += ",";  // Comma between treatments, not after last

}

content += "\n";

logCount++;

}

}

Serial.println(" Spectro content prepared: " + String(content.length())

+ " bytes for " + String(logCount) + " window entries");

// Single file operation if (createSDFile(filename, content)) {

if (logCount > 0) {

Serial.println(" Spectro data logged successfully!");

Serial.println("   File: " + filename);         Serial.println("   Window entries: " + String(logCount));         Serial.println("   Format:

Date,Time,Treatment1,Treatment2,Treatment3,Treatment4,Treatment5

");

} else {

Serial.println("ℹ No spectro data to log (header-only file created)");

}

} else {

Serial.println(" Spectro logging failed");

}

}

// Main logging function - Simplified for dual SPI (no conflicts!)

void logSensorDataToCSV() {     if (sensorCount == 0) return;

Serial.println(" Starting data logging session (dual SPI - zero interference)...");

Serial.println(" Historical buffer contains: " + String(historyCount) + " readings");

//  No SD.begin() needed - already initialized on HSPI bus!

// Log ALL historical temperature and humidity data     logTempHumidityData();

// Small delay     delay(100);

// Log ALL historical spectro data with treatment assignments     logSpectroData();

//  NEW: Clear the historical buffer after successful logging     clearDataHistory();

Serial.println(" Data logging session completed!");

}

// HTTP POST endpoint to receive sensor data from ESP32-C3

void handleData() {    if (server.method() != HTTP_POST) {

server.send(405, "text/plain", "Method Not Allowed");

return;

}

String body = server.arg("plain");

StaticJsonDocument<400> doc;

DeserializationError error = deserializeJson(doc, body);

if (error) {

Serial.println(" Invalid JSON: " + String(error.c_str()));      server.send(400, "text/plain", "Invalid JSON");

return;

}

String id = doc["id"];    float temp = doc["temp"];

float hum = doc["hum"];

int index = findSensorIndex(id);    if (index == -1 && sensorCount < MAX_SENSORS) {

index = sensorCount++;

sensors[index].id = id;

}

if (index != -1) {

// Update current display data (for TFT display)     sensors[index].temp = temp;     sensors[index].hum = hum;

sensors[index].lastUpdated = getTimeStamp();  // Use real-time timestamp

sensors[index].lastUpdateMillis = millis();  // Track update time for timeout detection

sensors[index].dataChanged = true;  // Mark this sensor as changed

// Mark that we've received data (for "No sensor data" message removal)     hasReceivedData = true;

displayNeedsUpdate = true;  // Update display when new data

arrives

// SD Card logging timer start (HSPI always ready)      if (lastLogTime == 0 && sdAvailable) {        lastLogTime = millis(); // Start 1-hour timer on first sensor data        Serial.println("⏱ SD Card logging timer started - first log in 1 hour");

}

// Handle spectro data and prepare for history buffer

int spectroData[6] = {0};      int spectroCount = 0;

bool lightsOn = areLightsOn(); // Check if lights are on (by schedule

OR manually)

if (doc.containsKey("spectro")) {        JsonArray spectro = doc["spectro"];

spectroCount = spectro.size();

// Only store spectro data if lights are on (scheduled OR manual)        if (lightsOn) {

sensors[index].spectroCount = spectroCount; // For display          for (int i = 0; i < spectroCount && i < 6; i++) {            spectroData[i] = spectro[i];

sensors[index].spectro[i] = spectro[i]; // For display

}

} else {

// Lights off - don't store spectro data for display or logging          sensors[index].spectroCount = 0;          spectroCount = 0; // Clear spectro count so it's not added to history

Serial.println("   Spectro data ignored (lights off and not manually enabled)");

}      } else {

sensors[index].spectroCount = 0; // none sent

}

//  NEW: Add this reading to historical data buffer (only if lights on for spectro)

// Always add temp/humidity, but only add spectro if lights are on      if (lightsOn || spectroCount == 0) {

addToDataHistory(id, temp, hum, spectroData, spectroCount);

} else {

// Only add temp/humidity when lights are off (no spectro)

addToDataHistory(id, temp, hum, spectroData, 0);

}

//  Print received data

Serial.println(" Data Received from " + id);

Serial.println("   Temp: " + String(temp) + "°C");

Serial.println("   Hum: " + String(hum) + "%");

if (sensors[index].spectroCount > 0) {        Serial.print("   Spectro: ");

for (int i = 0; i < sensors[index].spectroCount; i++) {          Serial.print(sensors[index].spectro[i]);

if (i < sensors[index].spectroCount - 1) Serial.print(", ");

}

Serial.println();

}

Serial.println("  ⏱ Time: " + sensors[index].lastUpdated);

Serial.println("-----------------------------");

server.send(200, "text/plain", "OK");

} else {

server.send(500, "text/plain", "Too many sensors");

}

}

// Web interface root handler   void handleRoot() {

String html = "<!DOCTYPE html><html><head>";     html += "<title>MushroomFarm Monitor</title>";     html += "<meta name=\"viewport\" content=\"width=device-width,

initial-scale=1\">";     html += "<style>";

html += "body { font-family: Arial; margin: 20px; background: #f0f8ff;

}";

html += ".container { max-width: 600px; margin: 0 auto; background: white; padding: 20px; border-radius: 10px; }";

html += ".status { padding: 10px; margin: 10px 0; border-radius: 5px;

}";

html += ".online { background: #d4edda; color: #155724; }";     html += ".offline { background: #f8d7da; color: #721c24; }";    html += "button { padding: 10px 20px; margin: 5px; border: none; border-radius: 5px; cursor: pointer; }";    html += ".sync-btn { background: #007bff; color: white; }";    html += ".sync-btn:hover { background: #0056b3; }";    html += ".relay-btn { background: #dc3545; color: white; font-weight:

bold; }";

html += ".relay-btn:hover { background: #c82333; }";    html += ".relay-btn.on { background: #28a745; }";    html += ".relay-btn.on:hover { background: #218838; }";    html += ".reset-btn { background: #ffc107; color: #000; font-weight:

bold; }";

html += ".reset-btn:hover { background: #e0a800; }";     html += "#time-info { font-size: 1.2em; margin: 15px 0; }";     html += ".sensor-table { width: 100%; border-collapse: collapse; margin: 15px 0; }";

html += ".sensor-table th, .sensor-table td { padding: 8px; border: 1px solid #ddd; text-align: center; }"; html += ".sensor-table th { background: #f8f9fa; }"; html += "</style></head><body>";

html += "<div class=\"container\">";

html += "<h1>MushroomFarm Monitor</h1>";

html += "<div class=\"status\">";

html += "<strong>RTC Status:</strong> ";     if (rtcAvailable) {

html += "<span class=\"online\">Online</span>";       html += "<div id=\"time-info\">Current Time: " + getDateTime() +

"</div>";     } else {

html += "<span class=\"offline\">Offline</span>";

}

html += "</div>";

html += "<div class=\"status\">";     html += "<strong>Auto Time Sync:</strong>";     html += "<button class=\"sync-btn\" onclick=\"syncTime()\">Sync Time Now</button>";

html += "<div id=\"sync-result\" style=\"margin-top: 10px;\"></div>";     html += "</div>";

html += "<div class=\"status\">";

html += "<strong>Connected Sensors:</strong> <span>" +

String(sensorCount) + "</span>";

html += "</div>";

html += "<div class=\"status\">";    html += "<strong>Data Logging:</strong> <span>" + String(sdAvailable ? "Active" : "Offline") + "</span>";    html += "</div>";

html += "<div class=\"status\">";    html += "<strong>MCP23017 Relays:</strong> <span>" + String(mcpAvailable ? "Available" : "Not Available") + "</span>";

html += "</div>";

html += "<h3>Manual Relay Control</h3>";    html += "<div style=\"margin: 15px 0;\">";    html += "<button class=\"relay-btn\" onclick=\"toggleRelay(1)\" id=\"relay1\">Mist Maker 1: OFF</button>";    html += "<button class=\"relay-btn\" onclick=\"toggleRelay(2)\" id=\"relay2\">Mist Maker 2: OFF</button>";    html += "<button class=\"relay-btn\" onclick=\"toggleRelay(3)\" id=\"relay3\">Mist Maker 3: OFF</button>";    html += "<br><br>";

html += "<button class=\"relay-btn\" onclick=\"toggleRelay(4)\" id=\"relay4\">Exhaust Fan: OFF</button>";    html += "<button class=\"relay-btn\" onclick=\"toggleRelay(5)\" id=\"relay5\">Sprinkler Pump: OFF</button>";    html += "<button class=\"relay-btn\" onclick=\"toggleRelay(7)\" id=\"relay7\">Lights: OFF</button>";    html += "</div>";    html += "<div style=\"margin: 15px 0; padding: 10px; background: #fff3cd; border-radius: 5px; border: 1px solid #ffc107;\">";    html += "<strong>  Manual Override Active:</strong> Relays controlled manually will not return to automatic mode until reset.";    html += "<br><br>";    html += "<button class=\"reset-btn\" onclick=\"resetOverrides()\">Reset to Automatic Control</button>";    html += "<div id=\"reset-result\" style=\"margin-top: 10px;\"></div>";    html += "</div>";

html += "<h3>Live Sensor Data</h3>";    html += "<table class=\"sensor-table\">";    html +=

"<tr><th>Sensor</th><th>Temperature</th><th>Humidity</th><th>Last

Update</th></tr>";

html += "</table>";

html += "<div style=\"margin-top: 30px; color: #666; font-size:

0.9em;\">";     html += "<p><strong>How Time Sync Works:</strong></p>";     html += "<ul>";     html += "<li>Auto-syncs on first page load only</li>";     html += "<li>Uses your device current time (automatic)</li>";     html += "<li>Converts to Manila time (UTC+8)</li>";     html += "<li>Updates RTC hardware clock</li>";     html += "<li>Use 'Sync Time Now' button for manual sync</li>";     html += "</ul></div></div>";

// JavaScript    html += "<script>";    html += "function toggleRelay(relayNum) {";    html += "const button = document.getElementById('relay' + relayNum);";    html += "const isCurrentlyOn = button.classList.contains('on');";    html += "const action = isCurrentlyOn ? 'off' : 'on';";    html += "fetch('/relay-control', {";    html += "method: 'POST',";    html += "headers: {'Content-Type': 'application/json'},";    html += "body: JSON.stringify({relay: relayNum, action: action})";    html += "}).then(response => response.text()).then(data => {";

html += "if (data === 'OK') {";    html += "if (action === 'on') {";    html += "button.classList.add('on');";    html += "button.textContent = button.textContent.replace(': OFF', ': ON');";    html += "} else {";    html += "button.classList.remove('on');";    html += "button.textContent = button.textContent.replace(': ON', ': OFF');";    html += "}";    html += "} else {";    html += "alert('Relay control failed: ' + data);";    html += "}";    html += "}).catch(error => {";    html += "alert('Relay control error: ' + error);";    html += "});";    html += "}";    html += "function resetOverrides() {";

html += "document.getElementById('reset-result').innerHTML =

'Resetting...';";    html += "fetch('/reset-overrides', {";    html += "method: 'POST',";    html += "headers: {'Content-Type': 'application/json'}";    html += "}).then(response => response.text()).then(data => {";    html += "document.getElementById('reset-result').innerHTML = data;";

html += "setTimeout(() => location.reload(), 2000);";    html += "}).catch(error => {";

html += "document.getElementById('reset-result').innerHTML =

'Reset failed: ' + error;";    html += "});";    html += "}";    html += "function syncTime() {";     html += "const now = new Date();";

html += "const manilaTime = new Date(now.getTime() + (8 * 60 * 60

* 1000));";     html += "const timeData = {";     html += "year: manilaTime.getUTCFullYear(),";     html += "month: manilaTime.getUTCMonth() + 1,";     html += "day: manilaTime.getUTCDate(),";     html += "hour: manilaTime.getUTCHours(),";     html += "minute: manilaTime.getUTCMinutes(),";

html += "second: manilaTime.getUTCSeconds()";

html += "};";

html += "document.getElementById('sync-result').innerHTML =

'Syncing...';";     html += "fetch('/sync-time', {";     html += "method: 'POST',";     html += "headers: {'Content-Type': 'application/json'},";     html += "body: JSON.stringify(timeData)";     html += "}).then(response => response.text()).then(data => {";     html += "document.getElementById('sync-result').innerHTML =

'Success: ' + data;";

html += "setTimeout(() => location.reload(), 2000);";     html += "}).catch(error => {";

html += "document.getElementById('sync-result').innerHTML = 'Sync failed';";     html += "});";     html += "}";

// Auto-sync only on first page load    html += "let hasAutoSynced = sessionStorage.getItem('hasAutoSynced');";    html += "window.onload = function() {";    if (rtcAvailable) {      html += "if (!hasAutoSynced) {";      html += "setTimeout(syncTime, 1000);";      html += "sessionStorage.setItem('hasAutoSynced', 'true');";

html += "}";

}

html += "};";

html += "</script></body></html>";

server.send(200, "text/html", html);

}

// Handle automatic time synchronization

void handleTimeSync() {     if (!rtcAvailable) {

server.send(400, "text/plain", "RTC not available");

return;

}

StaticJsonDocument<200> doc;

DeserializationError error = deserializeJson(doc, server.arg("plain"));

if (error) {

server.send(400, "text/plain", "Invalid JSON");

return;

}

int year = doc["year"]; int month = doc["month"]; int day = doc["day"]; int hour = doc["hour"]; int minute = doc["minute"]; int second = doc["second"];

// Validate time components if (year < 2020 || year > 2050 || month < 1 || month > 12 ||

day < 1 || day > 31 || hour < 0 || hour > 23 ||          minute < 0 || minute > 59 || second < 0 || second > 59) {       server.send(400, "text/plain", "Invalid time values");

return;

}

// Set RTC time

rtc.adjust(DateTime(year, month, day, hour, minute, second));

// Verify the time was set correctly

DateTime now = rtc.now();

String currentTime = getDateTime();

Serial.println(" Time synchronized via web interface:");

Serial.println("  Set: " + String(year) + "/" + String(month) + "/" +

String(day) +

" " + String(hour) + ":" + String(minute) + ":" +

String(second));

Serial.println("  RTC: " + currentTime);

server.send(200, "text/plain", "Time synced successfully! RTC: " + currentTime);

}

// Handle manual relay control from web interface  void handleRelayControl() {    if (!mcpAvailable) {

server.send(400, "text/plain", "MCP23017 not available");      return;

}

StaticJsonDocument<200> doc;

DeserializationError error = deserializeJson(doc, server.arg("plain"));

if (error) {

server.send(400, "text/plain", "Invalid JSON");

return;

}

int relayNum = doc["relay"];    String action = doc["action"];

// Validate relay number   if (relayNum < 1 || relayNum > 7) {     server.send(400, "text/plain", "Invalid relay number (1-7)");

return;

}

// Map relay numbers to MCP23017 pins and control functions

switch (relayNum) {      case 1: // Mist Maker 1        if (action == "on") {          onMistMakerOne();

mistMaker1ManualOverride = true;  // Set manual override flag

} else {          offMistMakerOne();

mistMaker1ManualOverride = true;  // Set manual override flag

}        break;

case 2: // Mist Maker 2        if (action == "on") {          onMistMakerTwo();

mistMaker2ManualOverride = true;  // Set manual override flag

} else {          offMistMakerTwo();

mistMaker2ManualOverride = true;  // Set manual override flag

}        break;

case 3: // Mist Maker 3        if (action == "on") {          onMistMakerThree();

mistMaker3ManualOverride = true;  // Set manual override flag

} else {          offMistMakerThree();

mistMaker3ManualOverride = true;  // Set manual override flag

}        break;

case 4: // Exhaust Fan        if (action == "on") {          onExhaustFan();          exhaustFanManualOverride = true;

} else {          offExhaustFan();

exhaustFanManualOverride = true;

}        break;

case 5: // Sprinkler Pump

if (action == "on") {          onSprinklerPump();          sprinklerPumpManualOverride = true;

} else {          offSprinklerPump();

sprinklerPumpManualOverride = true;

}        break;

case 7: // Lights       if (action == "on") {         onLights();         lightsManualOverride = true;

} else {

offLights();         lightsManualOverride = true;

}

break;

default:        server.send(400, "text/plain", "Invalid relay number");

return;

}

Serial.println(" Web relay control: Relay " + String(relayNum) + " " + action);

server.send(200, "text/plain", "OK");

}

// Handle reset of manual override flags from web interface  void handleResetOverrides() {

Serial.println("\n Resetting Manual Override Flags via Web

Interface:");

Serial.println("==========================================")

;

// Reset all manual override flags    mistMaker1ManualOverride = false;    mistMaker2ManualOverride = false;    mistMaker3ManualOverride = false;    exhaustFanManualOverride = false;    sprinklerPumpManualOverride = false;

lightsManualOverride = false;

Serial.println(" All manual overrides reset - automatic control restored");

Serial.println(" Relays will now respond to sensor readings and schedules");

Serial.println("==========================================\n

");

server.send(200, "text/plain", "Manual overrides reset! Automatic

control restored.");

}

// Draw system status header for 480x320 display   void drawSystemStatus() {

// Top status bar - larger text for 4" display     tft.setTextSize(2);

tft.setTextColor(TFT_WHITE, TFT_BLACK);

// WiFi Status     tft.setCursor(10, 10);

tft.print("WiFi: ");

tft.setTextColor(TFT_GREEN, TFT_BLACK);

tft.print(ssid);

tft.setTextColor(TFT_WHITE, TFT_BLACK);

tft.print(" (OK)");

// Real-Time Clock Reading     tft.setCursor(10, 35);

if (rtcAvailable) {

tft.setTextColor(TFT_CYAN, TFT_BLACK);

tft.print("Time: ");       tft.print(getDateTime());

} else {

tft.setTextColor(TFT_RED, TFT_BLACK);

tft.print("RTC: Not Available");

}

// SD Card Status (HSPI always available)     tft.setCursor(10, 60);

if (sdAvailable) {

tft.setTextColor(TFT_GREEN, TFT_BLACK);       tft.print("SD Card: HSPI OK - Instant logging");

} else {

tft.setTextColor(TFT_RED, TFT_BLACK);

tft.print("SD Card: HSPI Not Available");

}

}

// Update only a specific sensor row in temp/humidity table   void updateSensorRow(int sensorIndex, int rowY) {

if (sensorIndex >= sensorCount) return;

int rowHeight = 24;

// Clear the row area

tft.fillRect(11, rowY - 2, 458, rowHeight, TFT_BLACK);

// Sensor name - larger text     tft.setCursor(30, rowY);

tft.setTextSize(2);

tft.setTextColor(TFT_YELLOW, TFT_BLACK);     tft.print(sensors[sensorIndex].id);

// Temperature - larger text

tft.setCursor(180, rowY);     tft.setTextColor(TFT_GREEN, TFT_BLACK);     if (sensors[sensorIndex].temp > 0) {

tft.print(sensors[sensorIndex].temp, 1);       tft.print("C");

} else {

tft.setTextColor(TFT_DARKGREY, TFT_BLACK);

tft.print("--.-C");

}

// Humidity - larger text     tft.setCursor(340, rowY);     tft.setTextColor(TFT_BLUE, TFT_BLACK);     if (sensors[sensorIndex].hum > 0) {       tft.print(sensors[sensorIndex].hum, 1);

tft.print("%");

} else {

tft.setTextColor(TFT_DARKGREY, TFT_BLACK);

tft.print("--.-%%");

}

// Draw row separator if not the first row     if (sensorIndex > 0) {

tft.drawLine(10, rowY - 4, 470, rowY - 4, TFT_DARKGREY);

}

// Mark as updated     sensors[sensorIndex].dataChanged = false;

}

// Update only changed treatment columns in spectro mode   void updateTreatmentData() {

// Check if lights are on (by schedule OR manually) - if not, don't

update spectro display     if (!areLightsOn()) {

// Lights are off - refresh the "disabled" message if needed       displayNeedsUpdate = true;       displayInitialized = false;       return;

}

int colWidth = 460 / 5;

int rowY = 130;

// Check which sensors have changed and update their corresponding treatments

for (int i = 0; i < sensorCount; i++) {

if (!sensors[i].dataChanged) continue;

String sensorId = sensors[i].id;

// Update treatments based on which sensor changed      if (sensorId == "sensor1") {

// Treatment 1: Sensor 1 (spectro values 1-3) - Green        clearTreatmentColumn(0, colWidth, rowY);

displayTreatmentColumn(0, colWidth, rowY, 1, "sensor1", 0, 2,

TFT_GREEN);

// Treatment 2: Sensor 1 (spectro values 4-6) - Green        clearTreatmentColumn(1, colWidth, rowY);

displayTreatmentColumn(1, colWidth, rowY, 2, "sensor1", 3, 5,

TFT_GREEN);

}

else if (sensorId == "sensor2") {        // Treatment 3: Sensor 2 (spectro values 1-3) - Green        clearTreatmentColumn(2, colWidth, rowY);

displayTreatmentColumn(2, colWidth, rowY, 3, "sensor2", 0, 2,

TFT_GREEN);

// Treatment 4: Sensor 2 (spectro values 4-6) - Cyan        clearTreatmentColumn(3, colWidth, rowY);

displayTreatmentColumn(3, colWidth, rowY, 4, "sensor2", 3, 5,

TFT_CYAN);

}

else if (sensorId == "sensor3") {        // Treatment 5: Sensor 3 (spectro values 1-3) - Red        clearTreatmentColumn(4, colWidth, rowY);

displayTreatmentColumn(4, colWidth, rowY, 5, "sensor3", 0, 2,

TFT_RED);

}

sensors[i].dataChanged = false;

}

}

// Clear a specific treatment column for update   void clearTreatmentColumn(int columnIndex, int colWidth, int startY) {     int columnX = 10 + (columnIndex * colWidth);

tft.fillRect(columnX + 1, startY, colWidth - 1, 130, TFT_BLACK);

}

// Update only the system status bar (time and date) - optimized for minimal flicker   void updateSystemStatus() {

// Smart update strategy for 1-second refresh with minimal flicker

static String lastTimeDisplay = "";

String currentTimeDisplay = "";

if (rtcAvailable) {

currentTimeDisplay = "Time: " + getDateTime();

} else {

currentTimeDisplay = "RTC: Not Available";

}

// Only update if the display text actually changed     if (currentTimeDisplay != lastTimeDisplay) {

// Only clear the exact area where the time will be displayed (more precise)

tft.fillRect(120, 35, 340, 20, TFT_BLACK);  // Clear just the time portion

// Real-Time Clock Reading       tft.setCursor(10, 35);       tft.setTextSize(2);

if (rtcAvailable) {

tft.setTextColor(TFT_CYAN, TFT_BLACK);

tft.print("Time: ");         tft.print(getDateTime());

} else {

tft.setTextColor(TFT_RED, TFT_BLACK);

tft.print("RTC: Not Available");

}

lastTimeDisplay = currentTimeDisplay;

}

}

// Draw table with borders for 480x320 display (without TIME column)   void drawTable() {

// Clear table area - use full width of 480x320 screen

tft.fillRect(10, 90, 460, 190, TFT_BLACK);

// Draw table borders with thicker lines for better visibility     tft.drawRect(10, 90, 460, 190, TFT_WHITE);          // Outer border     tft.drawLine(10, 120, 470, 120, TFT_WHITE);         // Header separator

tft.drawLine(160, 90, 160, 280, TFT_WHITE);         // Column 1 separator

tft.drawLine(320, 90, 320, 280, TFT_WHITE);         // Column 2 separator

// Table headers - larger text size     tft.setTextSize(2);

tft.setTextColor(TFT_WHITE, TFT_BLACK);

tft.setCursor(50, 95);

tft.print("SENSOR");

tft.setCursor(180, 95);

tft.print("TEMPERATURE");

tft.setCursor(360, 95);

tft.print("HUMIDITY");

}

// Display temperature and humidity readings in table format for

480x320   void displayTempHumidity() {

tft.fillScreen(TFT_BLACK);

// Draw system status

drawSystemStatus();

// Draw table structure

drawTable();

// Fill table with sensor data - larger text and more space (in sorted order)     int rowY = 130;  // Start after header

int rowHeight = 24;  // More space between rows for larger text

// Get sorted sensor indices int sortedIndices[MAX_SENSORS]; int sortedCount; getSortedSensorIndices(sortedIndices, sortedCount); // Debug: Show sorted order

Serial.print(" Displaying sensors in sorted order: "); for (int i = 0; i < sortedCount && i < 6; i++) {       Serial.print(sensors[sortedIndices[i]].id);

if (i < sortedCount - 1 && i < 5) Serial.print(", ");

}

Serial.println();

int displayedRows = 0;

for (int i = 0; i < sortedCount && displayedRows < 6; i++) { // Display up to 6 sensors with larger text

int sensorIndex = sortedIndices[i];  // Use sorted index

// Skip stale sensors (not updated in 30+ seconds)       if (isSensorStale(sensorIndex)) {

continue;

}

// Draw row separator       if (displayedRows > 0) {

tft.drawLine(10, rowY - 4, 470, rowY - 4, TFT_DARKGREY);

}

// Sensor name - larger text       tft.setCursor(30, rowY);

tft.setTextSize(2);

tft.setTextColor(TFT_YELLOW, TFT_BLACK);

tft.print(sensors[sensorIndex].id);

// Temperature - larger text

tft.setCursor(180, rowY);       tft.setTextColor(TFT_GREEN, TFT_BLACK);       if (sensors[sensorIndex].temp > 0) {

tft.print(sensors[sensorIndex].temp, 1);         tft.print("C");

} else {

tft.setTextColor(TFT_DARKGREY, TFT_BLACK);

tft.print("--.-C");

}

// Humidity - larger text       tft.setCursor(340, rowY);       tft.setTextColor(TFT_BLUE, TFT_BLACK);       if (sensors[sensorIndex].hum > 0) {         tft.print(sensors[sensorIndex].hum, 1);

tft.print("%");

} else {

tft.setTextColor(TFT_DARKGREY, TFT_BLACK);

tft.print("--.-%%");

}

rowY += rowHeight;

displayedRows++;

}

// Only show "No sensor data" message if we haven't received any data yet

if (!hasReceivedData || displayedRows == 0) {       tft.setCursor(100, 180);

tft.setTextSize(2);       tft.setTextColor(TFT_RED, TFT_BLACK);

tft.print("No sensor data received yet");

}

// Button instruction at bottom - larger text     tft.fillRect(0, 285, 480, 35, TFT_NAVY);     tft.setCursor(10, 295);

tft.setTextSize(2);     tft.setTextColor(TFT_WHITE, TFT_NAVY);

tft.print("Press button to switch to Spectro Mode");

}

// Display treatments based on spectro sensor readings for 480x320   void displaySpectroReadings() {

tft.fillScreen(TFT_BLACK);

// Draw system status

drawSystemStatus();

// Check if lights are on - display spectro data when lights are on (by

schedule OR manually)

bool lightsOn = areLightsOn();

if (!lightsOn) {

// Lights are off - show message instead of spectro data       tft.fillRect(10, 90, 460, 190, TFT_BLACK);       tft.drawRect(10, 90, 460, 190, TFT_WHITE);       // Outer border

tft.setTextSize(2);

tft.setTextColor(TFT_YELLOW, TFT_BLACK);       tft.setCursor(50, 150);       tft.print("Spectro Data Collection");       tft.setCursor(80, 180);

tft.print("Disabled");

tft.setTextSize(1);       tft.setTextColor(TFT_CYAN, TFT_BLACK);       tft.setCursor(60, 220);       tft.print("Lights OFF Period");       tft.setCursor(80, 240);

tft.print("(7:00 AM - 4:00 PM)");

tft.setTextSize(1);       tft.setTextColor(TFT_WHITE, TFT_BLACK);       tft.setCursor(30, 260);

tft.print("Auto at 4:00 PM or turn on lights manually");

// Button instruction - larger text       tft.fillRect(0, 285, 480, 35, TFT_NAVY);       tft.setCursor(20, 295);

tft.setTextSize(2);       tft.setTextColor(TFT_WHITE, TFT_NAVY);       tft.print("Press button to switch to Temp Mode");

return;

}

// Lights are on - display spectro data normally     // Draw treatment table structure for larger screen     tft.fillRect(10, 90, 460, 190, TFT_BLACK);     tft.drawRect(10, 90, 460, 190, TFT_WHITE);       // Outer border

tft.drawLine(10, 120, 470, 120, TFT_WHITE);      // Header separator

// Create column separators for 5 treatments     int colWidth = 460 / 5;  // ~92 pixels per column

for (int i = 1; i < 5; i++) {       int x = 10 + (i * colWidth);

tft.drawLine(x, 90, x, 280, TFT_WHITE);

}

// Treatment headers - smaller text to fit    tft.setTextSize(1);

tft.setTextColor(TFT_WHITE, TFT_BLACK);

for (int i = 0; i < 5; i++) {      int centerX = 10 + (i * colWidth) + (colWidth / 2) - 30;      tft.setCursor(centerX, 95);

tft.printf("Treatment%d", i + 1);

}

// Display treatment data

int rowY = 130;

// Treatment 1: Sensor 1 (spectro values 1-3 = indices 0,1,2) - Green

displayTreatmentColumn(0, colWidth, rowY, 1, "sensor1", 0, 2,

TFT_GREEN);

// Treatment 2: Sensor 1 (spectro values 4-6 = indices 3,4,5) - Green

displayTreatmentColumn(1, colWidth, rowY, 2, "sensor1", 3, 5,

TFT_GREEN);

// Treatment 3: Sensor 2 (spectro values 1-3 = indices 0,1,2) - Green

displayTreatmentColumn(2, colWidth, rowY, 3, "sensor2", 0, 2,

TFT_GREEN);

// Treatment 4: Sensor 2 (spectro values 4-6 = indices 3,4,5) - Cyan

displayTreatmentColumn(3, colWidth, rowY, 4, "sensor2", 3, 5,

TFT_CYAN);

// Treatment 5: Sensor 3 (spectro values 1-3 = indices 0,1,2) - Red

displayTreatmentColumn(4, colWidth, rowY, 5, "sensor3", 0, 2,

TFT_RED);

// Button instruction - larger text    tft.fillRect(0, 285, 480, 35, TFT_NAVY);    tft.setCursor(20, 295);

tft.setTextSize(2);    tft.setTextColor(TFT_WHITE, TFT_NAVY);

tft.print("Press button to switch to Temp Mode");

}

// Helper function to display treatment column data  void displayTreatmentColumn(int columnIndex, int colWidth, int startY, int treatmentNum,

const char* sensorId, int startIdx, int endIdx, uint16_t

textColor) {

int columnX = 10 + (columnIndex * colWidth);

int centerX = columnX + (colWidth / 2);

// Find the sensor

int sensorIndex = findSensorIndex(String(sensorId));

// Check if sensor data is stale

if (sensorIndex != -1 && isSensorStale(sensorIndex)) {      // Show "No Data" for stale sensors      tft.setCursor(columnX + 15, startY + 20);      tft.setTextSize(1);

tft.setTextColor(TFT_DARKGREY, TFT_BLACK);

tft.print("Timeout");      return;

}

if (sensorIndex != -1 && sensors[sensorIndex].spectroCount > 0) {

// Display available spectro values      int valueY = startY;      int valueCount = 0;

// Determine range      int start = (startIdx == -1) ? 0 : startIdx;      int end = (endIdx == -1) ? sensors[sensorIndex].spectroCount - 1 : endIdx;

tft.setTextSize(1);      tft.setTextColor(textColor, TFT_BLACK);

for (int i = start; i <= end && i < sensors[sensorIndex].spectroCount; i++) {        if (valueY < 260) {  // Don't go too low          tft.setCursor(columnX + 5, valueY);          tft.printf("%d", sensors[sensorIndex].spectro[i]);          valueY += 15;

valueCount++;

}

if (valueCount >= 6) break;  // Limit to 6 values per column

}

if (valueCount == 0) {        // No data in range        tft.setCursor(columnX + 15, startY + 20);

tft.setTextColor(TFT_DARKGREY, TFT_BLACK);

tft.print("No Data");

}

} else {

// No sensor data available      tft.setCursor(columnX + 10, startY + 20);      tft.setTextSize(1);      tft.setTextColor(TFT_RED, TFT_BLACK);      tft.print("No Sensor");

}

}

// Handle button press with debouncing   void handleButton() {     int reading = digitalRead(BUTTON_PIN);

if (reading != lastButtonState) {       lastDebounceTime = millis();

Serial.println(" Button state change: " + String(lastButtonState ==

HIGH ? "HIGH->LOW" : "LOW->HIGH"));

}

if ((millis() - lastDebounceTime) > debounceDelay) {       if (reading != currentButtonState) {         currentButtonState = reading;

Serial.println(" Button debounced: " + String(currentButtonState ==

HIGH ? "RELEASED" : "PRESSED"));

if (currentButtonState == LOW) { // Button pressed (assuming pullup)           showSpectroMode = !showSpectroMode;           displayNeedsUpdate = true;  // Trigger display update

displayInitialized = false;  // Force full redraw when switching

modes

Serial.println(" Display mode switched to: " +

String(showSpectroMode ? "Spectro" : "Temp/Humidity"));

Serial.println(" Full display refresh triggered for mode switch");

}

}

}

lastButtonState = reading;

}

void setup() {     Serial.begin(115200);

Serial.println(" Starting Mushroom Farm Monitor - DUAL SPI

VERSION!");

//  Initialize HSPI for SD card FIRST

Serial.println(" Initializing HSPI bus for SD card...");

hspi.begin(14, 12, 13, 15); // SCK, MISO, MOSI, CS

// Initialize CS pins     pinMode(5, OUTPUT);          // TFT CS (VSPI)     pinMode(SD_CS_PIN, OUTPUT);  // SD CS (HSPI)      digitalWrite(5, HIGH);       // Disable TFT     digitalWrite(SD_CS_PIN, HIGH); // Disable SD

delay(100);

//  Initialize TFT on VSPI (automatic with TFT_eSPI)     Serial.println(" Initializing TFT Display on VSPI bus...");

tft.init();

tft.setRotation(3);  // Landscape orientation for 480x320     tft.fillScreen(TFT_BLACK);

// Show startup screen with larger text

tft.setTextColor(TFT_GREEN, TFT_BLACK);

tft.setTextSize(3);     tft.setCursor(50, 80);     tft.println("Dual SPI System");     tft.setTextSize(2);     tft.setCursor(120, 120);

tft.println("TFT: VSPI | SD: HSPI");

// Initialize DS3231 RTC     if (rtc.begin()) {       rtcAvailable = true;

// Check if RTC lost power        if (rtc.lostPower()) {

Serial.println(" RTC lost power - time may be incorrect");         Serial.println(" Use 'SET_TIME YYYY MM DD HH MM SS' to set correct time");

Serial.println(" Or use 'SYNC_TIME' for instructions");

}

Serial.println(" RTC Initialized");

Serial.println(" Current Time: " + getDateTime());

} else {

Serial.println(" DS3231 RTC not found - continuing without RTC");

rtcAvailable = false;

}

//  Initialize SD Card on HSPI bus     Serial.println(" Initializing SD Card on HSPI bus...");

sdAvailable = initializeSDCard();

if (sdAvailable) {

Serial.println(" SD Card initialized on separate HSPI bus!");       Serial.println(" ZERO SPI interference - TFT and SD fully independent!");

Serial.println(" Data logging: ENABLED and INSTANT");

Serial.println(" No warm-up period needed with dual SPI!");

Serial.println(" Parallel operations supported");

Serial.println(" Filename format: DDMmmYYYY-HHMM(temp/spectro).csv");

} else {

Serial.println(" SD Card initialization failed on HSPI");

Serial.println(" Check HSPI wiring (pins 12,13,14,15)");

}

// Initialize MCP23017 at I2C address 0x20 (for relay control)    if (mcp.begin_I2C(0x20)) {      mcpAvailable = true;

Serial.println(" MCP23017 initialized for relay control");

// Set all relay pins as OUTPUT     mcp.pinMode(RELAY1, OUTPUT);     mcp.pinMode(RELAY2, OUTPUT);     mcp.pinMode(RELAY3, OUTPUT);     mcp.pinMode(EXHAUST_FAN, OUTPUT);     mcp.pinMode(SPRINKLER_PUMP, OUTPUT);     mcp.pinMode(LIGHTS, OUTPUT);     mcp.pinMode(RELAY_C3_POWER, OUTPUT); // GPA7 - C3 power

control relay

// Initialize all relays to OFF (HIGH for active-low relay modules)

mcp.digitalWrite(RELAY1, HIGH);      mcp.digitalWrite(RELAY2, HIGH);      mcp.digitalWrite(RELAY3, HIGH);      mcp.digitalWrite(EXHAUST_FAN, HIGH);      mcp.digitalWrite(SPRINKLER_PUMP, HIGH);      mcp.digitalWrite(LIGHTS, HIGH);     mcp.digitalWrite(RELAY_C3_POWER, HIGH); // De-energized: NC connected (power ON)

Serial.println(" All relays initialized to OFF state");

Serial.println("   Mist Makers 1-3: GPA0, GPA1, GPA2");

Serial.println("   Exhaust Fan: GPA3 (≥92% humidity)");

Serial.println("   Sprinkler Pump: GPA4 (8:30 AM & 2:00 PM, 10s)");

Serial.println("   Lights: GPA5 (5:00 AM - 8:00 PM)");

Serial.println("   GPA6: (unused)");

Serial.println("   GPA7: C3 Power Relay (A7) initialized - power ON

by default");

} else {

Serial.println(" MCP23017 not found - relay control disabled");       mcpAvailable = false;

}

// Initialize button

Serial.println("Setting up button on pin 33...");

pinMode(BUTTON_PIN, INPUT_PULLUP);

// Initialize WiFi AP WiFi.softAP(ssid, password, 1, 0, 8); delay(1000); Serial.println(" AP Started");     Serial.print(" IP Address: ");

Serial.println(WiFi.softAPIP());

// Update TFT with initialization status     tft.setTextSize(2);

tft.setCursor(20, 160);     tft.setTextColor(TFT_CYAN, TFT_BLACK);

tft.print("WiFi AP: ");     tft.println(ssid);     tft.setCursor(20, 180);     tft.print("IP: ");

tft.println(WiFi.softAPIP());

tft.setCursor(20, 200);

if (rtcAvailable) {

tft.setTextColor(TFT_GREEN, TFT_BLACK);       tft.println("RTC: OK");

} else {

tft.setTextColor(TFT_RED, TFT_BLACK);

tft.println("RTC: Not Available");

}

tft.setCursor(20, 220);

if (sdAvailable) {

tft.setTextColor(TFT_GREEN, TFT_BLACK);       tft.println("SD Card: HSPI OK");

} else {

tft.setTextColor(TFT_RED, TFT_BLACK);

tft.println("SD Card: HSPI Failed");

}

// Start web server with multiple endpoints    // Note: /data endpoint disabled - sensors are now read locally    server.on("/data", HTTP_POST, handleData);    server.on("/", HTTP_GET, handleRoot);    server.on("/sync-time", HTTP_POST, handleTimeSync);    server.on("/relay-control", HTTP_POST, handleRelayControl);    server.on("/reset-overrides", HTTP_POST, handleResetOverrides);    server.begin();

Serial.println(" HTTP Server Ready (Web interface only - sensors

read locally)");

tft.setCursor(20, 240);     tft.setTextColor(TFT_GREEN, TFT_BLACK);

tft.println("HTTP Server Ready");

delay(3000); // Show startup info for 3 seconds

// Show initial display (temp/humidity mode)     displayNeedsUpdate = true;  // Trigger initial display

displayInitialized = false; // Force full initialization

Serial.println(" Display initialized - Temperature/Humidity mode");     Serial.println(" Smart partial updates enabled - only changed data refreshes");

Serial.println(" Sensors displayed in sorted order (sensor1, sensor2, sensor3, etc.)");

Serial.println("Press button on pin 33 to switch to Spectro mode");

Serial.println();

Serial.println(" DUAL SPI STATUS SUMMARY:");

Serial.println("  RTC Available: " + String(rtcAvailable ? " YES" : "

NO"));

Serial.println("  TFT Display:  VSPI Bus (Pins 23,19,18,5)");

Serial.println("  SD Card: " + String(sdAvailable ? " HSPI Bus (Pins

13,12,14,15)" : " HSPI Failed"));

Serial.println("  MCP23017 Relays: " + String(mcpAvailable ? " I2C (6 relays)" : " DISABLED"));

Serial.println("  Sensor Data Source: ESP32-C3 via WiFi (HTTP

POST)");

Serial.println("  Expected Sensors: sensor1, sensor2, sensor3");    Serial.println("  Logging Status: " + String(sdAvailable ? " INSTANT" :

" DISABLED"));

Serial.println("  SPI Conflicts:  ELIMINATED!");

Serial.println();

Serial.println(" Historical Data Buffer: " +

String(MAX_HISTORY_ENTRIES) + " entries");

Serial.println("   → Capacity: 1+ month of continuous operation");    Serial.println("   → DHT22: 3 sensors × 12 windows/hour = 36 readings/hour");

Serial.println("   → Spectro: 5 treatments × 3 readings/window × 12 windows/hour = 180 readings/hour");

Serial.println("   → Total: 216 readings/hour × 24 hours × 30 days =

~155,520 readings/month");

Serial.println("   → Logging: Once per hour (aggregated data)");    Serial.println("   → Spectro CSV Format:

Date,Time,Treatment1,Treatment2,Treatment3,Treatment4,Treatment5

");

Serial.println();

Serial.println(" Spectro Reading Ranges:");

Serial.println("   Sensor 1: Treatment 1 (505-510), Treatment 2 (520-

525)");

Serial.println("   Sensor 2: Treatment 3 (550-555), Treatment 4 (490495)");

Serial.println("   Sensor 3: Treatment 5 (620-625)");

Serial.println();

Serial.println(" Commands: LOG_NOW, BUFFER_STATUS,

LIST_FILES, SET_TIME, HELP");

Serial.println(" Web Interface: http://192.168.4.1 (auto-sync time, relay control)");

Serial.println("========================================");

}

void loop() {    server.handleClient();

// Control all relays based on sensor readings and time schedules

controlAllRelays();

// Handle Serial commands for debugging     if (Serial.available()) {

String command = Serial.readString();       command.trim();       command.toUpperCase();

if (command == "LOG_NOW") {

Serial.println("\n Manual Logging Test:");

Serial.println("==========================================")

;

if (sensorCount == 0) {

Serial.println(" No sensor data - creating test data for logging test");

// Create temporary test sensor data           sensorCount = 4;           sensors[0].id = "sensor1"; sensors[0].temp = 23.5; sensors[0].hum = 85.2; sensors[0].lastUpdated = "12:34:56"; sensors[0].spectroCount = 6;           sensors[1].id = "sensor2"; sensors[1].temp = 24.1; sensors[1].hum = 87.8; sensors[1].lastUpdated = "12:34:56"; sensors[1].spectroCount = 6;           sensors[2].id = "sensor3"; sensors[2].temp = 23.8; sensors[2].hum = 86.7; sensors[2].lastUpdated = "12:34:56"; sensors[2].spectroCount = 0;           sensors[3].id = "sensor4"; sensors[3].temp = 24.2; sensors[3].hum = 88.1; sensors[3].lastUpdated = "12:34:56"; sensors[3].spectroCount = 6;

// Add test spectro data           for (int i = 0; i < 6; i++) {             sensors[0].spectro[i] = 1000 + i;             sensors[1].spectro[i] = 2000 + i;

sensors[3].spectro[i] = 4000 + i;

}

Serial.println(" Test sensor data created");

}

//  NO TFT PAUSE NEEDED - Dual SPI eliminates all conflicts!         Serial.println(" Logging directly with dual SPI (zero interference)...");

// Perform SD logging with zero conflicts

logSensorDataToCSV();

Serial.println("==========================================\n

");

}

else if (command.startsWith("SET_TIME ")) {         // Format: SET_TIME YYYY MM DD HH MM SS         // Example: SET_TIME 2024 09 22 14 30 00

command.replace("SET_TIME ", "");

// Parse the time components         int timeValues[6];         int valueIndex = 0;

int startPos = 0;

for (int i = 0; i <= command.length() && valueIndex < 6; i++) {

if (i == command.length() || command.charAt(i) == ' ') {             if (i > startPos) {

timeValues[valueIndex] = command.substring(startPos,

i).toInt();               valueIndex++;

}             startPos = i + 1;

}

}

if (valueIndex == 6) {

Serial.println("\n Manual RTC Time Setting:");

Serial.println("==========================================")

;

Serial.println("Setting RTC to: " + String(timeValues[0]) + "/" +

String(timeValues[1]) + "/" + String(timeValues[2]) + " "

+

String(timeValues[3]) + ":" + String(timeValues[4]) + ":"

+ String(timeValues[5]));

setRTCTime(timeValues[0], timeValues[1], timeValues[2],

timeValues[3], timeValues[4], timeValues[5]);

Serial.println("==========================================\n

");

} else {

Serial.println("\n Invalid time format!");

Serial.println("Use: SET_TIME YYYY MM DD HH MM SS");

Serial.println("Example: SET_TIME 2024 09 22 14 30 00\n");

}

}

else if (command == "LIST_FILES") {

Serial.println("\n SD Card Files:");

Serial.println("==========================================")

;

if (SD.begin(SD_CS_PIN, hspi)) {           listSDCardFiles();

} else {

Serial.println(" SD card not accessible");

}

Serial.println("==========================================\n

");

}

else if (command == "BUFFER_STATUS") {

Serial.println("\n Historical Data Buffer Status:");

Serial.println("==========================================")

;

Serial.println("Buffer Size: " + String(MAX_HISTORY_ENTRIES) +

" entries");

Serial.println("Current Usage: " + String(historyCount) + " entries");

float percentFull = (historyCount * 100.0) /

MAX_HISTORY_ENTRIES;

Serial.println("Percentage Full: " + String(percentFull, 1) + "%");

// Show capacity estimates for 3-second intervals

Serial.println("\nCapacity Analysis (3-second intervals):");

Serial.println("  4 sensors = 80 entries/minute");

Serial.println("  5-minute period = 400 entries");

Serial.println("  Current buffer = " +

String(MAX_HISTORY_ENTRIES/80.0, 1) + " minutes capacity");

if (historyCount > 0) {           int estimatedMinutes = historyCount / 80;           int estimatedSeconds = (historyCount % 80) * 3 / 4;           Serial.println("  Current data span ≈ " + String(estimatedMinutes)

+ "m" + String(estimatedSeconds) + "s");

}

if (historyCount > 0) {           Serial.println("\nBuffer Contains:");

// Count entries per sensor           int sensorCounts[MAX_SENSORS] = {0};           String sensorNames[MAX_SENSORS];           int uniqueSensors = 0;

for (int i = 0; i < historyCount; i++) {             if (dataHistory[i].isValid) {

String sensorId = String(dataHistory[i].sensorId);

// Find or add sensor               int sensorIndex = -1;               for (int j = 0; j < uniqueSensors; j++) {                 if (sensorNames[j] == sensorId) {                   sensorIndex = j;                   break;

}

}

if (sensorIndex == -1 && uniqueSensors < MAX_SENSORS) {

sensorIndex = uniqueSensors;                 sensorNames[sensorIndex] = sensorId;

uniqueSensors++;

}

if (sensorIndex != -1) {

sensorCounts[sensorIndex]++;

}

}

}

for (int i = 0; i < uniqueSensors; i++) {             Serial.println("  " + sensorNames[i] + ": " +

String(sensorCounts[i]) + " readings");

}

// Show oldest and newest timestamps           if (historyCount > 0) {             Serial.println("\nTime Range:");             Serial.println("  Oldest: " + String(dataHistory[0].dateTime) + " "

+ String(dataHistory[0].timeOnly));

Serial.println("  Newest: " + String(dataHistory[historyCount1].dateTime) + " " + String(dataHistory[historyCount-1].timeOnly));

}

} else {

Serial.println("Buffer is empty");

}

Serial.println("==========================================\n

");

}

else if (command == "RESET_OVERRIDES") {

Serial.println("\nResetting Manual Override Flags:");

Serial.println("==========================================")

;

mistMaker1ManualOverride = false;        mistMaker2ManualOverride = false;        mistMaker3ManualOverride = false;        exhaustFanManualOverride = false;        sprinklerPumpManualOverride = false;        lightsManualOverride = false;

Serial.println("All manual overrides reset - automatic control restored");

Serial.println("==========================================\n

");

}

else if (command == "HELP") {         Serial.println("\n Available Commands:");

Serial.println("  LOG_NOW         - Force immediate data logging");

Serial.println("  BUFFER_STATUS   - Show data buffer status");

Serial.println("  LIST_FILES      - Show files on SD card");         Serial.println("  SET_TIME YYYY MM DD HH MM SS - Set RTC time");

Serial.println("  RESET_OVERRIDES - Reset manual override flags");

Serial.println("  HELP            - Show this help message");

Serial.println("\n Web Interface: http://192.168.4.1");

Serial.println("  - View sensor data");

Serial.println("  - Set time automatically from your device");

Serial.println("  - Manual relay control (with override protection)");

}

}

// Handle button presses for switching display modes     handleButton();

// Clear stale sensors periodically (check every 30 seconds)     static unsigned long lastStaleCheck = 0;     if (millis() - lastStaleCheck >= 30000) {       clearStaleSensors();

lastStaleCheck = millis();

}

// Monitor C3 devices and power-cycle if any expected C3 is stale     monitorC3AndCycleRelay();

// Check if lights status changed - refresh spectro display if in spectro

mode

// Uses areLightsOn() to detect both scheduled and manual light

changes

static bool lastLightsState = false;     bool currentLightsState = areLightsOn();     if (showSpectroMode && displayInitialized && (currentLightsState != lastLightsState)) {

Serial.println(" Lights status changed - refreshing spectro display");       displayNeedsUpdate = true;

displayInitialized = false; // Force full redraw

}

lastLightsState = currentLightsState;

// Smart display updates - only refresh what changed     if (displayNeedsUpdate) {

// Check if we need to switch display modes or do full initialization       if (!displayInitialized || currentDisplayMode != showSpectroMode) {         Serial.println(" Full display refresh needed");         if (showSpectroMode) {

displaySpectroReadings();

} else {           displayTempHumidity();

}

displayInitialized = true;

currentDisplayMode = showSpectroMode;

}

// Otherwise, just update the changed sensor data

else {

Serial.println(" Updating only changed sensor data");

if (showSpectroMode) {           // Update only changed treatment columns           updateTreatmentData();

} else {

// Update only changed sensor rows (in sorted order)           int rowY = 130;  // Start after header

int rowHeight = 24;

// Get sorted sensor indices           int sortedIndices[MAX_SENSORS];           int sortedCount;

getSortedSensorIndices(sortedIndices, sortedCount);

int displayedRows = 0;           for (int i = 0; i < sortedCount && displayedRows < 6; i++) {

int sensorIndex = sortedIndices[i];  // Use sorted index

// Skip stale sensors (they should have been cleared, but

double-check)             if (isSensorStale(sensorIndex)) {               // Clear the row if sensor is stale               int staleRowY = 130 + (displayedRows * rowHeight);

tft.fillRect(11, staleRowY - 2, 458, rowHeight, TFT_BLACK);

continue;

}

if (sensors[sensorIndex].dataChanged || sensors[sensorIndex].temp > 0 || sensors[sensorIndex].hum > 0) {               updateSensorRow(sensorIndex, rowY);

Serial.println(" Updated sensor: " + sensors[sensorIndex].id);

}

rowY += rowHeight;

displayedRows++;

}

// If we have no displayed rows but have received data, clear the

"No sensor data" message area           if (displayedRows == 0 && hasReceivedData) {

tft.fillRect(100, 178, 280, 25, TFT_BLACK); // Clear the message area

}

}

// Always update system status (time changes every second)         updateSystemStatus();

}

displayNeedsUpdate = false;  // Reset flag

}

// Update time display periodically (every 1 second)     unsigned long currentTime = millis();     if (displayInitialized && currentTime - lastTimeUpdate >=

TIME_UPDATE_INTERVAL) {

updateSystemStatus();       lastTimeUpdate = currentTime;

Serial.println("  Periodic time display update (1s interval)");

}

//  Log sensor data to SD card every 1 hour (INSTANT with dual

SPI!)     if (currentTime - lastLogTime >= LOG_INTERVAL) {       Serial.println(" 1-hour logging timer reached!");

Serial.println(" Dual SPI Logging Status:");

Serial.println("  SD Available: " + String(sdAvailable ? " YES (HSPI)"

: " NO"));

Serial.println("  Sensor Count: " + String(sensorCount));       Serial.println("  Time Since Last Log: " + String((currentTime - lastLogTime) / 1000) + " seconds");

if (historyCount > 0 && sdAvailable) {

Serial.println(" All conditions met - logging instantly with dual

SPI!");

//  INSTANT LOGGING - No TFT pause needed!         logSensorDataToCSV();

lastLogTime = currentTime;

Serial.println(" Data logging completed instantly!");

} else {         if (historyCount == 0) {

Serial.println(" No historical data to log yet");

}         if (!sdAvailable) {

Serial.println(" SD card not available on HSPI");

Serial.println(" Check HSPI wiring (pins 12,13,14,15)");

}

lastLogTime = currentTime; // Reset timer anyway

}

} else {

// Show countdown every 5 minutes       static unsigned long lastCountdown = 0;       if (currentTime - lastCountdown >= 300000) { // 5 minutes         unsigned long timeUntilLog = (LOG_INTERVAL - (currentTime - lastLogTime)) / 1000;         unsigned long minutesUntilLog = timeUntilLog / 60;         Serial.println("⏱ Next SD card log in: " + String(minutesUntilLog) +

" minutes");

lastCountdown = currentTime;

}

}

//  No SPI cleanup needed - dual SPI buses are independent!

delay(50); // Small delay for better responsiveness

}

// ================================

// MCP23017 RELAY CONTROL FUNCTIONS

// ================================

// Mist Maker 1 Control Functions  void onMistMakerOne() {

Serial.println(" [" + getTimeStamp() + "] Turning ON Mistmaker 1

(Manual Override)");    if (mcpAvailable) {

mcp.digitalWrite(RELAY1, LOW);  // Active LOW relay      mistMaker1State = true;

} else {

Serial.println(" MCP23017 not available - cannot control relay");

}

}

void offMistMakerOne() {

Serial.println(" [" + getTimeStamp() + "] Turning OFF Mistmaker 1

(Manual Override)");    if (mcpAvailable) {

mcp.digitalWrite(RELAY1, HIGH); // Active LOW relay      mistMaker1State = false;

} else {

Serial.println(" MCP23017 not available - cannot control relay");

}

}

// Mist Maker 2 Control Functions  void onMistMakerTwo() {

Serial.println(" [" + getTimeStamp() + "] Turning ON Mistmaker 2

(Manual Override)");    if (mcpAvailable) {

mcp.digitalWrite(RELAY2, LOW);  // Active LOW relay      mistMaker2State = true;

} else {

Serial.println(" MCP23017 not available - cannot control relay");

}

}

void offMistMakerTwo() {

Serial.println(" [" + getTimeStamp() + "] Turning OFF Mistmaker 2

(Manual Override)");    if (mcpAvailable) {

mcp.digitalWrite(RELAY2, HIGH); // Active LOW relay      mistMaker2State = false;

} else {

Serial.println(" MCP23017 not available - cannot control relay");

}

}

// Mist Maker 3 Control Functions   void onMistMakerThree() {

Serial.println(" [" + getTimeStamp() + "] Turning ON Mistmaker 3");     if (mcpAvailable) {

mcp.digitalWrite(RELAY3, LOW);  // Active LOW relay       mistMaker3State = true;

} else {

Serial.println(" MCP23017 not available - cannot control relay");

}

}

void offMistMakerThree() {

Serial.println(" [" + getTimeStamp() + "] Turning OFF Mistmaker 3");

if (mcpAvailable) {

mcp.digitalWrite(RELAY3, HIGH); // Active LOW relay       mistMaker3State = false;

} else {

Serial.println(" MCP23017 not available - cannot control relay");

}

}

// Exhaust Fan Control Functions   void onExhaustFan() {

Serial.println(" [" + getTimeStamp() + "] Turning ON Exhaust Fan

(High humidity detected)");

if (mcpAvailable) {

mcp.digitalWrite(EXHAUST_FAN, LOW);  // Active LOW relay

exhaustFanState = true;

} else {

Serial.println(" MCP23017 not available - cannot control relay");

}

}

void offExhaustFan() {

Serial.println(" [" + getTimeStamp() + "] Turning OFF Exhaust Fan");     if (mcpAvailable) {

mcp.digitalWrite(EXHAUST_FAN, HIGH); // Active LOW relay       exhaustFanState = false;

} else {

Serial.println(" MCP23017 not available - cannot control relay");

}

}

// Sprinkler Pump Control Functions   void onSprinklerPump() {

Serial.println(" [" + getTimeStamp() + "] Turning ON Sprinkler Pump

(10 second cycle)");

if (mcpAvailable) {

mcp.digitalWrite(SPRINKLER_PUMP, LOW);  // Active LOW relay

sprinklerPumpState = true;       sprinklerStartTime = millis();

} else {

Serial.println(" MCP23017 not available - cannot control relay");

}

}

void offSprinklerPump() {

Serial.println(" [" + getTimeStamp() + "] Turning OFF Sprinkler

Pump");

if (mcpAvailable) {

mcp.digitalWrite(SPRINKLER_PUMP, HIGH); // Active LOW relay

sprinklerPumpState = false;       sprinklerStartTime = 0;

} else {

Serial.println(" MCP23017 not available - cannot control relay");

}

}

// Lights Control Functions   void onLights() {

Serial.println(" [" + getTimeStamp() + "] Turning ON Lights

(Evening/Night mode)");     if (mcpAvailable) {

mcp.digitalWrite(LIGHTS, LOW);  // Active LOW relay

lightsState = true;

} else {

Serial.println(" MCP23017 not available - cannot control relay");

}

}

void offLights() {

Serial.println(" [" + getTimeStamp() + "] Turning OFF Lights (Day mode)");     if (mcpAvailable) {

mcp.digitalWrite(LIGHTS, HIGH); // Active LOW relay

lightsState = false;

} else {

Serial.println(" MCP23017 not available - cannot control relay");

}

}

// ================================   // RELAY CONTROL LOGIC

// ================================

// Check if current time (with RTC adjustment) matches sprinkler

schedule   bool issprinklerTime() {

if (!rtcAvailable) return false;

DateTime now = rtc.now();

// Adjust for 2-second RTC delay

DateTime adjustedTime = now + TimeSpan(0, 0, 0,

RTC_DELAY_SECONDS);

int hour = adjustedTime.hour();     int minute = adjustedTime.minute();

int second = adjustedTime.second();

bool is2PM = (hour == 14 && minute == 0);

return (is2PM);

}

// Check if current time (with RTC adjustment) is in lights-on period (4

PM to 7 AM)   bool isLightsTime() {

if (!rtcAvailable) return false;

DateTime now = rtc.now();

// Adjust for 2-second RTC delay

DateTime adjustedTime = now + TimeSpan(0, 0, 0,

RTC_DELAY_SECONDS);

int hour = adjustedTime.hour();

// Lights ON from 4 PM (16:00) to 7 AM (07:00) - spans midnight

// This means: hour >= 16 OR hour < 7

return (hour >= 16 || hour < 7);

}

// Check if lights are actually ON (either by schedule OR manually turned on)

// Use this for spectrometer display logic   bool areLightsOn() {     // Lights are on if:

// 1. Currently in scheduled lights-on time, OR     // 2. Lights were manually turned on (lightsState is true)     return isLightsTime() || lightsState;

}

// Control mist makers based on sensor readings, humidity and temperature thresholds  void controlMistMakers() {

// Control Mist Maker 1 - controlled by Sensor 1 (only if not manually overridden)    if (!mistMaker1ManualOverride) {      int sensor1Index = findSensorIndex("sensor1");      if (sensor1Index != -1 && !isSensorStale(sensor1Index)) {

float hum1 = sensors[sensor1Index].hum;

// Turn ON if humidity is below 84        bool shouldTurnOn = (hum1 < HUMIDITY_LOW);

// Turn OFF if humidity is at or above 86 (prevents rapid cycling)

bool shouldTurnOff = (hum1 >= HUMIDITY_HIGH);

if (!mistMaker1State && shouldTurnOn) {          Serial.println(" [" + getTimeStamp() + "] Auto-turning ON Mistmaker 1 (Sensor 1 humidity: " + String(hum1) + "%)");

if (mcpAvailable) {            mcp.digitalWrite(RELAY1, LOW);

mistMaker1State = true;

}

} else if (mistMaker1State && shouldTurnOff) {

Serial.println(" [" + getTimeStamp() + "] Auto-turning OFF Mistmaker 1 (Sensor 1 humidity: " + String(hum1) + "%)");

if (mcpAvailable) {            mcp.digitalWrite(RELAY1, HIGH);

mistMaker1State = false;

}

}

} else if (sensor1Index != -1 && isSensorStale(sensor1Index) && mistMaker1State) {

// Turn off if sensor data is stale

Serial.println(" [" + getTimeStamp() + "] Sensor 1 timeout - turning

OFF Mistmaker 1");

if (mcpAvailable) {          mcp.digitalWrite(RELAY1, HIGH);

mistMaker1State = false;

}

}

}

// Control Mist Maker 2 - controlled by Sensor 2 only (only if not

manually overridden)    if (!mistMaker2ManualOverride) {      int sensor2Index = findSensorIndex("sensor2");      if (sensor2Index != -1 && !isSensorStale(sensor2Index)) {

float hum2 = sensors[sensor2Index].hum;

// Turn ON if humidity is below 84        bool shouldTurnOn = (hum2 < HUMIDITY_LOW);

// Turn OFF if humidity is at or above 86 (prevents rapid cycling)

bool shouldTurnOff = (hum2 >= HUMIDITY_HIGH);

if (!mistMaker2State && shouldTurnOn) {          Serial.println("[" + getTimeStamp() + "] Auto-turning ON Mistmaker 2 (Sensor 2 humidity: " + String(hum2) + "%)");

if (mcpAvailable) {            mcp.digitalWrite(RELAY2, LOW);

mistMaker2State = true;

}

} else if (mistMaker2State && shouldTurnOff) {

Serial.println("[" + getTimeStamp() + "] Auto-turning OFF Mistmaker 2 (Sensor 2 humidity: " + String(hum2) + "%)");

if (mcpAvailable) {            mcp.digitalWrite(RELAY2, HIGH);

mistMaker2State = false;

}

}

} else if (sensor2Index != -1 && isSensorStale(sensor2Index) && mistMaker2State) {

// Turn off if sensor data is stale

Serial.println(" [" + getTimeStamp() + "] Sensor 2 timeout - turning

OFF Mistmaker 2");

if (mcpAvailable) {          mcp.digitalWrite(RELAY2, HIGH);

mistMaker2State = false;

}

}

}

// Control Mist Maker 3 - controlled by Sensor 3     int sensor3Index = findSensorIndex("sensor3");     if (sensor3Index != -1 && !isSensorStale(sensor3Index)) {       float hum3 = sensors[sensor3Index].hum;

// Turn ON if humidity is below 84       bool shouldTurnOn = (hum3 < HUMIDITY_LOW);

// Turn OFF if humidity is at or above 86 (prevents rapid cycling)

bool shouldTurnOff = (hum3 >= HUMIDITY_HIGH);

if (!mistMaker3State && shouldTurnOn) {         Serial.println(" [" + getTimeStamp() + "] Auto-turning ON Mistmaker 3 (Sensor 3 humidity: " + String(hum3) + "%)");         onMistMakerThree();

} else if (mistMaker3State && shouldTurnOff) {

Serial.println(" [" + getTimeStamp() + "] Auto-turning OFF Mistmaker 3 (Sensor 3 humidity: " + String(hum3) + "%)");

offMistMakerThree();

}

} else if (sensor3Index != -1 && isSensorStale(sensor3Index) && mistMaker3State) {

// Turn off if sensor data is stale

Serial.println(" [" + getTimeStamp() + "] Sensor 3 timeout - turning

OFF Mistmaker 3");

offMistMakerThree();

}

}

void controlExhaustFan() {    if (!exhaustFanManualOverride) {      bool needsExhaust = false;

// Check all sensors for temperature outside range (skip stale

sensors)

for (int i = 0; i < sensorCount; i++) {

// Skip sensors with stale data (not updated in 30+ seconds)

if (isSensorStale(i)) {

continue;

}

if (sensors[i].temp > TEMP_EXHAUST_HIGH || sensors[i].temp <

TEMP_EXHAUST_LOW) {          needsExhaust = true;

Serial.println(" Temperature out of range detected: " + sensors[i].id +

" (Temp: " + String(sensors[i].temp) + "°C)");

break;

}

}

// Control exhaust fan based on temperature      if (needsExhaust && !exhaustFanState) {        onExhaustFan();

} else if (!needsExhaust && exhaustFanState) {

offExhaustFan();

}

}

}

// Control sprinkler pump based on schedule (8:30 AM and 2:00 PM,

10 seconds each)  void controlSprinklerPump() {

// Handle sprinkler duration timeout (turn off after 10 seconds)

if (sprinklerPumpState && (millis() - sprinklerStartTime >=

SPRINKLER_DURATION)) {

offSprinklerPump();

return;

}

// Check if it's sprinkler time    if (issprinklerTime()) {

// Start sprinkler if not already running and we haven't run this session

if (!sprinklerPumpState && !sprinklerActiveThisSession) {        onSprinklerPump();

sprinklerActiveThisSession = true;

}

} else {

// Reset session flag when not in sprinkler time window      sprinklerActiveThisSession = false;

}

}

// Control lights based on time schedule (5 AM to 8 PM)  void controlLights() {

if (!lightsManualOverride) { // <-- ADD THIS WRAPPER      bool shouldLightsBeOn = isLightsTime();      if (shouldLightsBeOn && !lightsState) {        onLights();

} else if (!shouldLightsBeOn && lightsState) {

offLights();

}

} // <-- ADD THIS CLOSING BRACE

}

// Master control function for all relays   void controlAllRelays() {

if (!mcpAvailable) return;  // Skip if MCP not available

controlMistMakers();    // Humidity-based mist control     controlExhaustFan();    // Humidity-based exhaust     controlSprinklerPump(); // Time-based sprinkler

controlLights();        // Time-based lights

}

// Monitor ESP32-C3 device timeouts and cycle their power using the

MCP23017 A7 relay

// Non-blocking: energize relay (LOW) for RELAY_OFF_DURATION ms then de-energize (HIGH) void monitorC3AndCycleRelay() {

if (!mcpAvailable) return;

// If currently in a cycle, finish it when duration elapsed   if (relayCycleActive) {

if (millis() - relayCycleStart >= RELAY_OFF_DURATION) {       // De-energize relay -> restore NC connection (power ON)

mcp.digitalWrite(RELAY_C3_POWER, HIGH);       relayCycleActive = false;       lastRelayCycleMillis = millis();

Serial.println(" [" + getTimeStamp() + "] C3 power relay:

RESTORED after cycle");

}     return;

}

// Enforce cooldown between cycles to avoid continuous toggling

if (millis() - lastRelayCycleMillis < RELAY_CYCLE_COOLDOWN_MS) return;

// Check expected C3 devices and trigger cycle if any is stale/missing   for (int i = 0; i < EXPECTED_C3_COUNT; i++) {     String id = String(expectedC3Ids[i]);     int idx = findSensorIndex(id);     if (isSensorStale(idx)) {

Serial.println(" [" + getTimeStamp() + "] C3 timeout detected for " +

id + " - cycling power (" + String(RELAY_OFF_DURATION/1000) +

"s)");

// Energize relay (active LOW) to open NC contact and cut power

mcp.digitalWrite(RELAY_C3_POWER, LOW);       relayCycleActive = true;       relayCycleStart = millis();       return;

}

} }